Flood Risk Assessment - Nyando Sub-County, Kisumu County, Kenya

A DEM-Based Hydrological Analysis for Disaster Risk Reduction

Prepared by: Daisy Tania

Date: July 2026

1. Introduction

1.1 Background

Nyando Sub-County is located in Kisumu County, western Kenya, along the lower reaches of the Nyando River. The region is characterized by low-lying topography (elevation 1,100–1,400m) and flat terrain, making it highly susceptible to seasonal flooding during heavy rainfall periods. Flooding in Nyando affects agricultural productivity, displaces communities, and damages infrastructure, yet spatially explicit flood risk data remains limited for local planning.

1.2 Study Objective

To identify flood-prone areas in Nyando Sub-County and estimate the population at risk using DEM-based hydrological analysis.

1.3 Study Area

Nyando Sub-County (approximately 603 km²) was selected for its documented flood history, flat topography, and availability of high-resolution spatial data. The area is predominantly rural with mixed agricultural land use and scattered settlements along the river corridor.

2. Data and Methods

2.1 Data Sources

| Dataset | Source | Resolution | Format | Year |
| --- | --- | --- | --- | --- |
| Digital Elevation Model (DEM) | OpenTopography | 30m | GeoTIFF | 2023 |
| Population | WorldPop | 100m | GeoTIFF | 2020 |
| Rivers | OpenStreetMap | Vector | Shapefile | 2023 |
| Administrative Boundary | GADM | Vector | Shapefile | 2022 |

2.2 Software

All processing was conducted in QGIS 3.44.11 LTR (Solothurn) with GDAL 3.12.3.

2.3 Methodology

2.3.1 Pre-processing

The DEM was downloaded using the OpenTopography DEM Downloader plugin and clipped to the Nyando Sub-County boundary. All data were reprojected to EPSG:32736 (WGS 84 / UTM Zone 36S) for metric-based calculations.

2.3.2 Hydrological Analysis

Sinks in the DEM were filled using the Wang & Liu algorithm (Fill Sinks). Flow direction was calculated to determine water movement patterns. Due to tool limitations in QGIS LTR, formal flow accumulation was not performed; instead, flood-prone areas were identified using a proxy approach combining elevation, slope, and river proximity.

2.3.3 Flood-Prone Area Identification

Three criteria were combined: • Low elevation: Areas below 1,200m (floodplain threshold) • Flat slope: Areas with slope < 3° (poor drainage) • River proximity: Within 1.5 km of the Nyando River channel

Flood-prone raster cells were converted to polygons and intersected with the river buffer to create realistic flood zones.

2.3.4 Vulnerability Assessment

WorldPop 2020 population data were overlaid on flood zones using Zonal Statistics to estimate population at risk per zone.

3. Results

3.1 Flood Zone Identification

One major flood zone was identified in Nyando Sub-County:

| Zone ID | Area (km²) | Population at Risk | Population Density (people/km²) | Risk Level |
| --- | --- | --- | --- | --- |
| Zone 06 | 106 | 64,088 | 604 | HIGH |

3.2 Key Findings

- Total flood-prone area: 106 km²

- Total population at risk: 64,088 people

- Population density in flood zone: 604 people/km²

- The flood zone is concentrated along the Nyando River corridor, reflecting the flat, low-lying topography of the sub-county

3.3 Spatial Distribution

The flood zone extends along the main Nyando River channel and its tributaries, covering agricultural land and settlement areas. The high population density (604 people/km²) indicates significant human exposure to flood hazard.

- Discussion

4.1 Interpretation of Results

The identification of a single, large flood zone (106 km²) with over 64,000 people at risk highlights the concentrated vulnerability of Nyando Sub-County. The flat terrain and low elevation along the river corridor create natural conditions for water accumulation during peak flows.

4.2 Comparison with Known Flood Events

Nyando has experienced documented flooding in 2026. The spatial extent of the identified flood zone aligns with reported inundation areas, particularly along the lower Nyando River.

4.3 Limitations

- The 30m DEM resolution may miss micro-topographic features that influence local flood patterns

- The absence of formal flow accumulation modeling (due to tool limitations) means the analysis relies on elevation and slope proxies rather than hydrological simulation

- Population data (WorldPop 2020) provides estimates rather than exact counts and may not reflect 2026 distributions

- Seasonal variation in river discharge and rainfall intensity are not accounted for

- Land cover was not incorporated; urbanization and agricultural practices influence runoff but were not modeled

4.4 Policy Implications

- Priority should be given to Zone 06 for flood early warning systems and evacuation planning

- Flood-resilient housing and infrastructure standards should be enforced in the 106 km² flood zone

- River buffer restoration (reforestation, wetland conservation) could reduce flood risk upstream

- Community-based flood monitoring should be established given the high population exposure

- Conclusion

This analysis identified a 106 km² flood-prone zone in Nyando Sub-County with an estimated 64,088 people at risk. The DEM-based approach, while limited by tool constraints, provides a spatially explicit foundation for disaster risk reduction planning. Future work should incorporate higher-resolution DEMs (e.g., Copernicus DEM 10m), hydrological modeling (e.g., HEC-RAS), and real-time rainfall data to improve flood forecasting accuracy. The methodology can be adapted for other flood-prone regions in western Kenya, including Budalangi (Busia County) and Kano Plains (Kisumu County).

- References

- OpenTopography. (2023). DEM Downloader Plugin. https://opentopography.org

- WorldPop. (2020). Population Data for Kenya. https://www.worldpop.org

- OpenStreetMap Contributors. (2023). River Network Data. https://www.openstreetmap.org

- QGIS Development Team. (2026). QGIS Geographic Information System,

Version 3.44 LTR. Open Source Geospatial Foundation.