Waste Collection Route Optimization: Kisumu, Kenya

Daisy Tania | July 2026

1. Project Overview

This project analyzes waste collection coverage across Kisumu Central, East, and West constituencies in Kisumu, Kenya. Using OpenStreetMap road networks and building footprints, I identified existing collection point clusters, calculated service coverage areas, and proposed new locations to reduce access gaps.

2. Methodology

| Step | Tool/Method | Description |
| --- | --- | --- |
| Data acquisition | QuickOSM, GADM | Roads, buildings, administrative boundaries |
| Point generation | Building centroids + grid aggregation | 13,105 building points → 4,017 grid centroids → 803 final collection points |
| Coverage analysis | Buffer (1km, dissolved) | Service areas around existing points |
| Gap identification | Vector Difference | Uncovered areas = total area − service area |
| Proposed points | Grid-based gap analysis + building buffer filter | 14 new points in underserved zones |

3. Key Findings

| Metric | Value |
| --- | --- |
| Study area | 3 constituencies (Kisumu Central, East, West) |
| Existing collection points | 803 |
| Proposed new points | 14 |
| Service radius | 1,000 meters |
| Total coverage | 69% |
| Coverage gap | 31% |

4. Analysis

The 1km service radius analysis reveals that 69% of the study area falls within walking or vehicle access of existing waste collection infrastructure. The remaining 31% gap is concentrated in:

- Peripheral western zones — low road density, sparse buildings

- Southern fringe areas — industrial/agricultural land with minimal residential development

- Lake Victoria shoreline — excluded from point placement due to non-serviceable water zones

Adding 14 strategically placed collection points in the largest gap areas would marginally improve coverage. However, the modest gain indicates that universal coverage would require substantial investment (50–100+ additional points) in road infrastructure and collection logistics.

5. Recommendations

- Prioritize gap zones in Kisumu West — largest contiguous uncovered area

- Coordinate with road development — new collection points need accessible roads

- Consider mobile collection for remote areas — fixed points may not be cost-effective for sparse populations

- Reassess service radius — 1km is realistic for vehicle-based collection; 500m would require 2–3× more infrastructure.

6. Limitations

- Building data from OpenStreetMap may be incomplete in informal settlements

- 1km radius assumes vehicle access; pedestrian-only areas may need smaller radii

- Population density not weighted — all buildings treated equally

- Lake Victoria shoreline included in boundary but excluded from point placement

7. Tools Used

QGIS 3.x

OpenStreetMap / QuickOSM

GADM administrative boundaries

EPSG:32736 (UTM Zone 36S)