Solar Farm Site Suitability Analysis — Turkana County, Kenya

Date: June 2026

Tools: QGIS 3.44 (Solothurn), GDAL 3.12

CRS: EPSG:32737 — WGS 84 / UTM Zone 37S

Author: Daisy Tania

1. Introduction

Turkana County in northwestern Kenya is one of the sunniest regions in East Africa, with Global Horizontal Irradiance (GHI) values consistently exceeding 5.5 kWh/m²/day. Despite its solar energy potential, the county remains largely energy-poor. This project identifies areas most suitable for utility-scale solar farm development in Turkana using a multi-criteria weighted overlay approach in QGIS, with a focus on the Loima/Lodwar sub-region.

2. Objectives

- Identify areas of high solar irradiance within Turkana County

- Assess land suitability based on slope, land cover, and solar potential

- Produce a classified suitability map highlighting excellent candidate sites

- Estimate the total area of high-suitability land available for solar development

3. Data Sources

| Dataset | Source | Resolution | Year |
| --- | --- | --- | --- |
| Global Horizontal Irradiance (GHI) | Global Solar Atlas | ~1km | 2020 |
| Digital Surface Model (DSM) | OpenTopography DEM Downloader plugin | 30m | 2023 |
| Land Cover | ESA WorldCover | 10m | 2021 |
| Administrative Boundaries | GADM | Vector | 2024 |
| Roads | OpenStreetMap | Vector | 2026 |

4. Methodology

Phase 1 — Data Acquisition & Preparation

Downloaded GHI raster, DSM, and ESA WorldCover tiles for Turkana

Clipped all datasets to Turkana County boundary (GADM admin shapefile)

Reprojected all layers to EPSG:32737 for consistent metric measurements

Phase 2 — Criteria Development

Three suitability criteria were developed and scored on a 1–5 scale:

- Slope (weight: 30%)

Derived from Copernicus DSM using QGIS Slope tool

Reclassified: 0–3° = 5 (excellent), 3–5° = 4, 5–10° = 3, 10–15° = 2, >15° = 1

- Solar Irradiance (weight: 45%)

GHI raster clipped to Turkana

Reclassified: >6.0 kWh/m²/day = 5, 5.5–6.0 = 4, 5.0–5.5 = 3, 4.5–5.0 = 2, <4.5 = 1

- Land Cover (weight: 25%)

ESA WorldCover classes reclassified by suitability:

Bare/sparse (60) = 5, Grassland (30) = 4, Shrubland (20) = 3

Cropland (40) = 2, Tree cover/Built-up/Water/Wetland = 1

Phase 3 — Raster Alignment

All three suitability rasters aligned to a common extent, resolution (30m), and CRS using QGIS Warp (Reproject)

Nearest neighbor resampling used to preserve discrete 1–5 class values

Phase 4 — Weighted Overlay

Final suitability calculated in QGIS Raster Calculator:

(Solar × 0.45) + (Slope × 0.3125) + (LandCover × 0.25)

Output classified into four categories:

1.0–2.0 = Unsuitable, 2.0–3.0 = Marginal, 3.0–4.0 = Good, 4.0–5.0 = Excellent

Phase 5 — Site Extraction

Excellent sites (score ≥ 4.0) extracted via Raster Calculator and Polygonise

Small polygon artifacts removed (area < 2 km²)

Geometry validated using Fix Geometries tool

Total area calculated using Field Calculator ($area / 1,000,000)

5. Results

The weighted overlay analysis identified approximately 7,000 km² of excellent-suitability land within the Loima sub-region of Turkana County as having excellent solar farm suitability (score ≥ 4.0). These sites are concentrated primarily in the Loima/Lodwar sub-region, characterized by:

Flat to gently sloping terrain (0–5°)

High GHI values (5.5–6.5+ kWh/m²/day)

Bare or sparse vegetation land cover — minimal clearing required

The final suitability map shows a clear spatial pattern with excellent and good sites dominating the western and central parts of Turkana, while the eastern mountainous terrain scores lower due to slope constraints.

6. Discussion

Findings: The scale of excellent-suitability land within Loima alone (~7,000 km²) suggests significant untapped solar energy potential in Turkana, consistent with Kenya's Vision 2030 renewable energy goals and the county's existing energy infrastructure projects.

Limitations:

- Roads and protected areas were considered during project design but excluded from the final weighted overlay due to data extent issues; these should be incorporated in future iterations for a more complete suitability assessment

- Analysis is entirely desktop-based with no field validation; ground-truthing of top-ranked sites is recommended before any development decision

- GHI data resolution (~1km) is coarser than slope (30m) and land cover (10m), which may introduce minor spatial mismatches in the overlay

- DSM (Digital Surface Model) includes vegetation/building heights, which may slightly overestimate slope in vegetated areas — a bare-earth DEM (DTM) would be more accurate for terrain analysis.

Future improvements:

- Incorporate distance to power grid and transmission lines as a criterion

- Add protected areas buffer as a hard exclusion mask

- Validate with higher-resolution terrain data (ALOS DEM, 12.5m)

- Extend analysis to neighboring counties for regional comparison

7. Conclusion

This analysis demonstrates that Turkana County holds extensive land suitable for utility-scale solar farm development, with approximately 7,000 km² scoring in the excellent suitability range in the sub-region alone. The Loima/Lodwar sub-region emerges as a priority area based on a combination of high solar irradiance, flat terrain, and low-conflict land cover. This project serves as a baseline spatial framework that can be refined with additional criteria and field validation to support evidence-based renewable energy planning in northern Kenya.

8. References

Global Solar Atlas (2020). Solar resource data for Kenya. World Bank Group. https://globalsolaratlas.info

ESA WorldCover (2021). 10m global land cover map. European Space Agency. https://esa-worldcover.org

GADM (2024). Database of Global Administrative Areas. https://gadm.org

OpenStreetMap contributors (2026). Kenya road network. https://openstreetmap.org