URBAN HEAT ISLAND & GREEN SPACE EQUITY ANALYSIS

Nairobi City, Kenya

Landsat-Based Thermal and Vegetation Assessment for Environmental Justice

Prepared by: Daisy Tania

Date: July 2026

- Introduction

1.1 Background

Nairobi, Kenya's capital and largest city, exhibits one of the most pronounced urban heat islands in East Africa. The city's rapid urbanization, unequal green space distribution, and varied land use create stark microclimates. Affluent neighborhoods such as Westlands and Kilimani benefit from extensive tree cover, parks, and private gardens, while informal settlements including Kibera and Mathare experience minimal vegetation cover and elevated temperatures. This environmental inequality has direct implications for public health, energy consumption, and quality of life.

1.2 Study Objective

To quantify the relationship between green space (NDVI) and land surface temperature (LST) across Nairobi constituencies, identifying environmental injustice hotspots where high heat exposure coincides with low vegetation cover.

1.3 Study Area

Nairobi City County (approximately 696 km²), with focus on 17 constituencies representing diverse socioeconomic and land use contexts — from leafy suburbs to dense informal settlements and industrial zones.

- Data and Methods

2.1 Data Sources

| Dataset | Source | Resolution | Date | Format |
| --- | --- | --- | --- | --- |
| Landsat 8/9 OLI/TIRS | USGS Earth Explorer | 30 m | February 2023 | GeoTIFF |
| Nairobi Constituencies | GADM / [your source] | Vector | 2022 | Shapefile |

2.2 Software

QGIS 3.44.11 LTR (Solothurn) with GDAL 3.12.3.

2.3 Methodology

2.3.1 Pre-processing

Landsat 8/9 Level-1 imagery was downloaded for February 2023 (dry season, <10% cloud cover). The thermal band (Band 10) was converted from Digital Numbers to Land Surface Temperature (°C) using radiance scaling factors and Planck's equation constants from image metadata:

LST = (K2 / ln((K1 / ((DN × 0.00038) + 0.1)) + 1)) − 273.15

Where K1 = 799.0284 and K2 = 1329.2405 for Landsat 8 Band 10.

NDVI was calculated from red (Band 4) and near-infrared (Band 5) bands:

NDVI = (NIR − Red) / (NIR + Red)

2.3.2 Classification

NDVI was classified into green space categories: • High green space: NDVI > 0.3 • Moderate green space: 0.15–0.3 • Low green space: < 0.15

LST was classified into heat categories: • Cool: < 28°C • Moderate: 28–32°C • Hot: 32–36°C • Very hot: > 36°C

2.3.3 Injustice Hotspot Identification

Areas classified as "very hot" (>36°C) AND "low green space" (<0.15) were identified as environmental injustice hotspots using raster overlay analysis.

2.3.4 Zonal Statistics

Mean LST and mean NDVI were calculated for each Nairobi constituency to enable comparative analysis.

- Results

3.1 Overall Statistics

| Parameter | Value |
| --- | --- |
| Minimum LST | 24.14°C |
| Maximum LST | 49.42°C |
| LST range | 25.28°C |
| Minimum NDVI | −0.078 |
| Maximum NDVI | 0.563 |
| Mean NDVI | ~0.15 |

3.2 Constituency Comparison

| Constituency | Mean LST (°C) | Mean NDVI | Category |
| --- | --- | --- | --- |
| Westlands | 31.79 | 0.264 | Cool, green |
| Dagoretti North | 31.42 | 0.218 | Coolest, moderate green |
| Starehe | 34.40 | 0.116 | Warm, low green |
| Kibra | 33.32 | 0.169 | Warm, low green |
| Mathare | 35.86 | 0.127 | Warm, low green |
| Dagoretti South | 35.15 | 0.205 | Warm, moderate green |
| Roysambu | 36.11 | 0.217 | Hot, moderate green |
| Langata | 36.71 | 0.183 | Hot, moderate green |
| Ruaraka | 36.87 | 0.155 | Hot, low green |
| Kamukunji | 36.28 | 0.144 | Hot, low green |
| Embakasi West | 36.92 | 0.103 | Hot, low green |
| Embakasi South | 37.59 | 0.089 | Hot, very low green |
| Embakasi North | 38.02 | 0.109 | Hot, low green |
| Makadara | 38.41 | 0.108 | Hot, low green |
| Embakasi Central | 37.87 | 0.078 | Hot, very low green |
| Embakasi East | 42.22 | 0.106 | Very hot, low green |
| Kasarani | 42.50 | 0.127 | Hottest, low green |

3.3 Key Findings

• Temperature range of 25.28°C across the city — from 24.14°C (forests/parks) to 49.42°C (bare concrete/industrial roofs) • Coolest constituency: Dagoretti North (31.42°C, NDVI 0.218) • Hottest constituency: Kasarani (42.50°C, NDVI 0.127) • Greenest constituency: Westlands (NDVI 0.264, LST 31.79°C) • Least green constituency: Embakasi Central (NDVI 0.078, LST 37.87°C)

3.4 Environmental Injustice Hotspots

Seven constituencies classified as injustice hotspots (LST >36°C AND NDVI <0.15):

| Constituency | LST (°C) | NDVI | Population at Risk |
| --- | --- | --- | --- |
| Embakasi Central | 37.87 | 0.078 | High-density residential |
| Embakasi East | 42.22 | 0.106 | Industrial/residential |
| Embakasi North | 38.02 | 0.109 | High-density residential |
| Embakasi South | 37.59 | 0.089 | High-density residential |
| Embakasi West | 36.92 | 0.103 | High-density residential |
| Kasarani | 42.50 | 0.127 | Mixed residential |
| Makadara | 38.41 | 0.108 | Industrial/residential |

These areas experience extreme heat with minimal vegetation cover, disproportionately affecting low-income residents.

- Discussion

4.1 Interpretation of Results

The analysis reveals a clear inverse relationship between green space and temperature across Nairobi. Affluent constituencies (Westlands, Dagoretti North) maintain lower temperatures through extensive tree cover and planned green infrastructure. Conversely, Embakasi constituencies and Kasarani — characterized by dense informal housing, industrial activity, and minimal vegetation — experience dangerous heat levels exceeding 42°C.

The 25.28°C temperature range within a single city is remarkable and underscores the severity of Nairobi's urban heat island effect. For context, a 1°C increase in ambient temperature is associated with increased mortality, heat stress, and energy demand for cooling.

4.2 Comparison with Known Patterns

- Westlands/Kilimani: Confirmed as cool, green enclaves (31–32°C, NDVI >0.25)

- Kibera: Moderate temperature (33.32°C) despite low green space — possibly due to building density creating shade corridors

- Embakasi: The "industrial belt" shows the worst conditions — high impervious surfaces, minimal tree cover, and extreme heat

- Karura Forest effect: Dagoretti North's cool temperatures likely influenced by proximity to Karura Forest (though forest itself not separately zoned)

4.3 Limitations

- 30m Landsat resolution cannot capture individual trees or small parks

- Single image (February 2023) — seasonal variation not accounted for

- Level-1 data required atmospheric correction; Level-2 surface temperature product would improve accuracy

- Socioeconomic data (income, housing quality) not directly incorporated

- Population density within constituencies varies; mean values may mask intra-constituency disparities

4.4 Policy Implications

- Priority greening: Embakasi Central, Embakasi South, and Kasarani require urgent tree-planting and green infrastructure

- Urban planning: Mandate minimum green space ratios in new developments

- Informal settlement upgrading: Integrate shade trees and community gardens in Kibera, Mathare, and Embakasi

- Heat action plans: Early warning systems for days exceeding 40°C in hotspot constituencies

- Equity framework: Environmental justice should guide climate adaptation investments

- Conclusion

This analysis quantified extreme environmental inequality in Nairobi, where constituencies separated by just a few kilometers experience temperature differences of over 11°C. The identification of seven injustice hotspots — primarily in Embakasi and Kasarani — provides an evidence base for targeted climate adaptation. The methodology, combining freely available Landsat imagery with open administrative boundaries, is replicable for other African cities facing similar urban heat challenges. Future work should incorporate Sentinel-2 (10m) for finer green space mapping, building footprint data for shade analysis, and household surveys to link thermal conditions with lived experience.

- References

- USGS Earth Resources Observation and Science Center. (2023). Landsat 8/9

Collection 2 Level-1 Data. https://earthexplorer.usgs.gov

- GADM. (2022). Database of Global Administrative Areas, Version 4.1.

https://gadm.org

- QGIS Development Team. (2026). QGIS Geographic Information System, Version 3.44 LTR. Open Source Geospatial Foundation.

- NASA Landsat Science. (2023). Landsat 8 Data Users Handbook. https://www.usgs.gov/landsat-missions/landsat-8-data-users-handbook

- Kenya National Bureau of Statistics. (2019). 2019 Kenya Population and Housing Census. https://www.knbs.or.ke